| EOS-S19 | 16CH Mode1 | DMX Value | Function |
| --- | --- | --- | --- |
| Dimmer | 1 | 0---255 | Darkness----->Bright |
| Strobe | 2 | 0---7 8---63 64---71 72---127 128---133 134---189 190--195 196--250 251--255 | Undefined Fast Strobe----->SlowStrobe Undefined Fast Pulsation Slow Close--->Fast Pulsation Fast Close Undefined Slow Pulsation Fast Close--->Fast Pulsation Fast Close Undefined Random Strobe Undefined |
| Red | 3 | 0---255 | Darkness----->Bright |
| Green | 4 | 0---255 | Darkness----->Bright |
| Blue | 5 | 0---255 | Darkness----->Bright |
| White | 6 | 0---255 | Darkness----->Bright |
| Focus | 7 | 0---255 |  |
| Focus Rotation | 8 | 0---127 128---191 192---255 | 0°----->360° CW Rotation Fast---->Slow CCW Rotation Slow--->Fast |
| Pan | 9 | 0---255 |  |
| Pan FINE | 10 | 0---255 |  |
| Tilte | 11 | 0---255 |  |
| Tilte FINE | 12 | 0---255 |  |
| P/T Speed | 13 | 0---128 129---170 172---211 212---255 | Fastest Speed----->Slowest Speed Slow Speed(1/ 3 maxspeed) Medium Speed(2/ 3 maxspeed) Fast Speed |
| Control | 14 | 0---127 128---255 | Undefined Reset ALL |
| Macro | 15 | 0---10 11---21 22---32 33---43 44---54 55---65 66---76 77---87 88---98 99---109 110---120 121---131 132---143 144---154 155--- 165 166--- 176 177--- 187 188--- 198 199--- 209 210---220 221--- 231 232--- 242 243---255 | Undefine Macro1 Macro2 Macro3 Macro4 Macro5 Macro6 Macro7 Macro8 Macro9 Macro10 Macro11 Macro12 Macro13 Macro14 Macro15 Macro16 Macro17 Macro18 Macro19 Macro20 Macro21 Macro22 |
| MacroSpeed | 16 | 0---255 | Slowest--->Fastest |

| 19颗15W蜂眼 | 88CH Mode2 | DMX Value | Function |
| --- | --- | --- | --- |
| Dimmer | 1 | 0---255 | Darkness----->Bright |
| Strobe | 2 | 0---7 8---63 64---71 72---127 128---133 134---189 190--195 196--250 251--255 | Undefined Fast Strobe----->SlowStrobe Undefined Fast Pulsation Slow Close--->Fast Pulsation Fast Close Undefined SlowPulsationn Fast Close--->Fast Pulsation Fast Close Undefined Random Strobe Undefined |
| Red1 | 3 | 0---255 | Darkness----->Bright |
| Green1 | 4 | 0---255 | Darkness----->Bright |
| Blue1 | 5 | 0---255 | Darkness----->Bright |
| White1 | 6 | 0---255 | Darkness----->Bright |
| Red2 | 7 | 0---255 | Darkness----->Bright |
| Green2 | 8 | 0---255 | Darkness----->Bright |
| Blue2 | 9 | 0---255 | Darkness----->Bright |
| White2 | 10 | 0---255 | Darkness----->Bright |
| Red3 | 11 | 0---255 | Darkness----->Bright |
| Green3 | 12 | 0---255 | Darkness----->Bright |
| Blue3 | 13 | 0---255 | Darkness----->Bright |
| White3 | 14 | 0---255 | Darkness----->Bright |
| Red4 | 15 | 0---255 | Darkness----->Bright |
| Green4 | 16 | 0---255 | Darkness----->Bright |
| Blue4 | 17 | 0---255 | Darkness----->Bright |
| White4 | 18 | 0---255 | Darkness----->Bright |
| Red5 | 19 | 0---255 | Darkness----->Bright |
| Green5 | 20 | 0---255 | Darkness----->Bright |
| Blue5 | 21 | 0---255 | Darkness----->Bright |
| White5 | 22 | 0---255 | Darkness----->Bright |
| Red6 | 23 | 0---255 | Darkness----->Bright |
| Green6 | 24 | 0---255 | Darkness----->Bright |
| Blue6 | 25 | 0---255 | Darkness----->Bright |
| White6 | 26 | 0---255 | Darkness----->Bright |
| Red7 | 27 | 0---255 | Darkness----->Bright |
| Green7 | 28 | 0---255 | Darkness----->Bright |
| Blue7 | 29 | 0---255 | Darkness----->Bright |
| White7 | 30 | 0---255 | Darkness----->Bright |
| Red8 | 31 | 0---255 | Darkness----->Bright |
| Green8 | 32 | 0---255 | Darkness----->Bright |
| Blue8 | 33 | 0---255 | Darkness----->Bright |
| White8 | 34 | 0---255 | Darkness----->Bright |
| Red9 | 35 | 0---255 | Darkness----->Bright |
| Green9 | 36 | 0---255 | Darkness----->Bright |
| Blue9 | 37 | 0---255 | Darkness----->Bright |
| White9 | 38 | 0---255 | Darkness----->Bright |
| Red10 | 39 | 0---255 | Darkness----->Bright |
| Green10 | 40 | 0---255 | Darkness----->Bright |
| Blue10 | 41 | 0---255 | Darkness----->Bright |
| White10 | 42 | 0---255 | Darkness----->Bright |
| Red11 | 43 | 0---255 | Darkness----->Bright |
| Green11 | 44 | 0---255 | Darkness----->Bright |
| Blue11 | 45 | 0---255 | Darkness----->Bright |
| White11 | 46 | 0---255 | Darkness----->Bright |
| Red12 | 47 | 0---255 | Darkness----->Bright |
| Green12 | 48 | 0---255 | Darkness----->Bright |
| Blue12 | 49 | 0---255 | Darkness----->Bright |
| White12 | 50 | 0---255 | Darkness----->Bright |
| Red13 | 51 | 0---255 | Darkness----->Bright |
| Green13 | 52 | 0---255 | Darkness----->Bright |
| Blue13 | 53 | 0---255 | Darkness----->Bright |
| White13 | 54 | 0---255 | Darkness----->Bright |
| Red14 | 55 | 0---255 | Darkness----->Bright |
| Green14 | 56 | 0---255 | Darkness----->Bright |
| Blue14 | 57 | 0---255 | Darkness----->Bright |
| White14 | 58 | 0---255 | Darkness----->Bright |
| Red15 | 59 | 0---255 | Darkness----->Bright |
| Green15 | 60 | 0---255 | Darkness----->Bright |
| Blue15 | 61 | 0---255 | Darkness----->Bright |
| White15 | 62 | 0---255 | Darkness----->Bright |
| Red16 | 63 | 0---255 | Darkness----->Bright |
| Green16 | 64 | 0---255 | Darkness----->Bright |
| Blue16 | 65 | 0---255 | Darkness----->Bright |
| White16 | 66 | 0---255 | Darkness----->Bright |
| Red17 | 67 | 0---255 | Darkness----->Bright |
| Green17 | 68 | 0---255 | Darkness----->Bright |
| Blue17 | 69 | 0---255 | Darkness----->Bright |
| White17 | 70 | 0---255 | Darkness----->Bright |
| Red18 | 71 | 0---255 | Darkness----->Bright |
| Green18 | 72 | 0---255 | Darkness----->Bright |
| Blue18 | 73 | 0---255 | Darkness----->Bright |
| White18 | 74 | 0---255 | Darkness----->Bright |
| Red19 | 75 | 0---255 | Darkness----->Bright |
| Green19 | 76 | 0---255 | Darkness----->Bright |
| Blue19 | 77 | 0---255 | Darkness----->Bright |
| White19 | 78 | 0---255 | Darkness----->Bright |
| Focus | 79 | 0---255 |  |
| Focus Rotation | 80 | 0---127 128---191 192---255 | 0°----->360° CW Rotation Fast---->Slow CCW Rotation Slow--->Fast |
| Pan | 81 | 0---255 |  |
| Pan FINE | 82 | 0---255 |  |
| Tilte | 83 | 0---255 |  |
| Tilte FINE | 84 | 0---255 |  |
| P/T Speed | 85 | 0---128 129---170 172---211 212---255 | Fastest Speed----->Slowest Speed Slow Speed(1/ 3 maxspeed) Medium Speed(2/ 3 maxspeed) Fast Speed |
| Control | 86 | 0---127 128---255 | Undefined Reset ALL |
| Macro | 87 | 0---10 11---21 22---32 33---43 44---54 55---65 66---76 77---87 88---98 99---109 110---120 121---131 132---143 144---154 155--- 165 166--- 176 177--- 187 188--- 198 199--- 209 210---220 221--- 231 232--- 242 243---255 | Undefine Macro1 Macro2 Macro3 Macro4 Macro5 Macro6 Macro7 Macro8 Macro9 Macro10 Macro11 Macro12 Macro13 Macro14 Macro15 Macro16 Macro17 Macro18 Macro19 Macro20 Macro21 Macro22 |
| MacroSpeed | 88 | 0---255 | Slowest--->Fastest |