| ARE--FB5 | 20CH Mode1 | 17CH Mode2 | DMX Value | Funciton |
| --- | --- | --- | --- | --- |
| Dimmer | 1 | 1 | 0---255 | Darkness----->Bright |
| Dimmer FINE | 2 |  | 0---255 |  |
| Strobe | 3 | 2 | 0---3 4---103 104---107 108---207 208---212 213---225 226---238 239---251 252---255 | Closed Slow Strobe--->Fast Strobe Open Slow Pulsation--->Fast Pulsation Open Random Slow Strobe Random Medium Strobe Random Fast Strobe Open |
| Color | 4 | 3 | 0---5 6---12 13---19 20---26 27---33 34---40 41---47 48---54 55---61 62---68 69---76 77---83 84---90 91---97 98---104 105---111 112---118 119---127 128---134 135---141 142---148 149---155 156---205 206----255 | White Color1+White Color1 Color1 + Color2 Color2 Color2 + Color3 Color3 Color3 + Color4 Color4 Color4 + Color5 Color5 Color5 + Color6 Color6 Color6 + Color7 Color7 Color7 + Color8 Color8 Color8+Color9 Color9 Color9+Color10 Color10 Color10+WHITE CW Rotation Slow--->Fast CCW Rotation Fast--->Slow |
| Fixed Gobo | 5 | 4 | 0---3 4---7 8---11 12---15 16---19 20---23 24---27 28---31 32---35 36---39 40---43 44---76 77---88 89---121 122---134 135---147 148---160 161---173 174---186 187---199 200---213 214---226 227---239 240---255 | WHITE GOBO 1 GOBO 2 GOBO 3 GOBO 4 GOBO 5 GOBO 6 GOBO 7 GOBO 8 GOBO 9 GOBO10 CW SlowRotation------>FastRotation WHITE CCW SlowRotation------> FastRotation GOBO 1 Shake Slow---->Fast GOBO 2 Shake Slow---->Fast GOBO 3 Shake Slow---->Fast GOBO 4 Shake Slow---->Fast GOBO 5 Shake Slow---->Fast GOBO 6 Shake Slow---->Fast GOBO 7 Shake Slow---->Fast GOBO 8 Shake Slow---->Fast GOBO 9 Shake Slow---->Fast GOBO 10 Shake Slow---->Fast |
| Gobo1 Select | 6 | 5 | 0---7 8--- 15 16---23 24--- 31 32--- 39 40--- 47 48---59 60---80 81---101 102--- 122 123--- 143 144---164 165---189 190--- 222 223--- 255 | WHITE GOBO 1 GOBO 2 GOBO 3 GOBO 4 GOBO 5 GOBO 6 GOBO 1 Shake Slow---->Fast GOBO 2 Shake Slow---->Fast GOBO 3 Shake Slow---->Fast GOBO 4 Shake Slow---->Fast GOBO 5 Shake Slow---->Fast GOBO 6 Shake Slow---->Fast CW SlowRotation------>FastRotation CCW SlowRotation------> FastRotation |
| GOBO1 index Rotation | 7 | 6 | 0---127 128---191 192---255 | Position 0°---->360°(FINE in CH8) CW SlowRotation------> FastRotation CCWFastRotation------>SlowRotation |
| GOBO1 indexRotation FINE | 8 |  | 0---255 | GOBO1 index Rotation FINE |
| PRISM1 | 9 | 7 | 0---127 128--255 | EXCLUDED INSERTED |
| PRISM1 Rotation | 10 | 8 | 0---127 128---190 191---194 195---255 | Position 0°---->360° CW FastRotation------> SlowRotation STOP CCW Slow Rotation------>Fast Rotation |
| FORST | 11 | 9 | 0---127 128---255 | EXCLUDED INSERTED |
| FOCUS | 12 | 10 | 0---255 |  |
| FOCUS FINE | 13 |  | 0---255 |  |
| PAN(X) | 14 | 11 | 0---255 |  |
| PAN(X) FINE | 15 | 12 | 0---255 |  |
| TILTE(Y) | 16 | 13 | 0---255 |  |
| TILTE(Y) FINE | 17 | 14 | 0---255 |  |
| P/T SPEED | 18 | 15 | 0---128 129---170 171---211 212---255 | Fastest------->Slowest Slow Speed ( 1/3 MaxSpeed) Medium Speed ( 1/2 MaxSpeed) Fast Speed (Fastest Speed) |
| Lamp Control | 19 | 16 | 0---32 33---64 65---96 97---128 129---160 161---193 194---225 226---255 | Undefined Undefined Undefined Undefined Undefined Undefined Undefined Undefined |
| Control | 20 | 17 | 0---127 128---255 | Undefined Reset ALL( >4s) |